RETAIN DSS

Technical Reference Manual

Decision Support System for PVC/PET Tarpaulin Recycling

RETAIN EU Project  —  Grant 101181056

Work Package 3  —  Task T3.4

May 2026

# 1. Introduction

The RETAIN DSS (Decision Support System) is a Python-based computational tool developed within Task T3.4 of the RETAIN EU project (Grant 101181056). Its purpose is to assist recycling process engineers and researchers in identifying the optimal operating conditions for three alternative routes for recycling end-of-life PVC/PET composite tarpaulins, such as truck covers, agricultural sheets, and event structures.

The system integrates three computational layers:

(1) Synthetic database. Three datasets of 500 records each, one per recycling route, encoding physically plausible correlations between material properties, process parameters, and performance outcomes.

(2) Data-driven surrogate models. A bank of 20 XGBoost regressors trained on the synthetic datasets, capable of predicting all technical and economic performance indicators given any combination of material inputs and process parameters in milliseconds.

(3) Multi-objective optimizer. An NSGA-II genetic algorithm that searches the process parameter space of each route independently to find Pareto-optimal trade-offs among five simultaneous objectives: PVC purity, mass yield, material quality, total energy consumption, and net economic margin.

This document provides a complete technical description of all variables, equations, model configurations, and algorithmic choices embedded in the tool. Each section combines plain-language explanations suitable for non-specialist readers with the formal mathematical definitions required for full technical reproducibility.

# 2. Input Material Properties

The eight material descriptors listed below characterize the physicochemical state of the PVC/PET tarpaulin as received at the recycling facility, before any process step. They are identical for all three recycling routes because they describe the feedstock, not the process.

| Variable | Unit | Range | Description |
| --- | --- | --- | --- |
| pvc_content | %wt | 40 – 70 | Mass fraction of poly(vinyl chloride) polymer in the composite tarpaulin, as a percentage of total dry weight. |
| pet_content | %wt | 20 – 45 | Mass fraction of poly(ethylene terephthalate) fibres constituting the woven or knitted fabric scrim embedded within the PVC coating. |
| additive_content | %wt | 5 – 20 | Mass fraction of functional additives compounded into the PVC matrix: plasticisers, heat stabilisers, UV absorbers, flame retardants, and pigments. |
| contamination_level | %wt | 0 – 15 | Mass fraction of extraneous materials not part of the original tarpaulin (soil, oils, adhesives, biological matter, metal fragments). |
| particle_size_d50 | mm | 0.5 – 50 | Median particle diameter of the pre-shredded input material (d50 of the particle size distribution by mass). |
| moisture_content | %wt | 0 – 8 | Water absorbed by the tarpaulin assembly, primarily in the PET fibre network and at the PVC–PET interface. |
| material_age | years | 1 – 20 | Estimated service lifetime before collection, used as a proxy for cumulative thermal and UV degradation of the polymer chains. |
| tensile_strength_input | MPa | 50 – 300 | Ultimate tensile strength of the composite tarpaulin as received, encoding the combined effect of PET fibre integrity and PVC degradation state. |

## 2.1 Detailed Parameter Definitions

PVC content (pvc_content)

Poly(vinyl chloride) constitutes the primary structural matrix and the coating layer of industrial tarpaulins, typically present at 40–70 %wt. Higher PVC fractions generally favor routes that target PVC recovery directly (Routes 1 and 2), where it is mechanically separated or selectively dissolved. This variable is the primary driver of revenue for all three routes.

PET content (pet_content)

The woven or knitted scrim of poly(ethylene terephthalate) fibres provides tensile and tear resistance. PET content is broadly anti-correlated with PVC content. High-performance technical tarpaulins (heavy-duty truck covers, stadium roofing membranes) tend toward higher PET fractions (35–45 %wt), while agricultural films lean toward higher PVC.

Additive content (additive_content)

Functional additives compounded into the PVC matrix serve specific performance roles. Plasticizers (typically di-isononyl phthalate, DINP, or citrate esters, at 10–50 parts per hundred resin) impart flexibility. Heat stabilizers (Ca/Zn complexes, organotin compounds, or legacy lead systems) prevent thermal degradation during processing. UV absorbers and pigments control weathering and color. This fraction is the primary economic target of Route 3 (selective additive extraction), since recovered plasticizers and stabilizers command significantly higher market prices than bulk recycled polymer.

Contamination level (contamination_level)

This variable encodes the mass fraction of extraneous materials accumulated during service life and end-of-life handling that are not part of the original tarpaulin composition. Contaminants include: mineral matter (soil, clay, sand from ground contact in agricultural and construction applications); hydrocarbon residues (oils and greases from machinery contact in transport tarpaulins); adhesive and paint residues (from labelling, repair patches, or overspray); biological matter (mold, organic debris in agricultural use); and metallic fragments (grommets, staples, wire eyelets). Contamination is the single strongest negative predictor of output purity across all three routes.

Median input particle size (particle_size_d50)

The d50 is the particle diameter at which 50% of the total mass is finer, characterizing the particle size distribution of the pre-shredded feedstock. Large values (>20 mm) indicate only a coarse primary shredding has been applied; small values (<5 mm) indicate fine pre-grinding. For Route 1, this variable sets the starting point for secondary milling. For Routes 2 and 3, it controls the specific surface area available for solvent or extractant contact, directly affecting dissolution and extraction kinetics.

Moisture content (moisture_content)

Water absorbed primarily in the PET fiber network and at the PVC–PET interface during outdoor storage, rainfall exposure, washing operations, or recovery from flooded sites. Moisture negatively impacts PVC purity (dilution and incomplete dissolution in Routes 2 and 3) and mechanical recycling yield (steam expansion and sticking during milling in Route 1). Values above 5 %wt indicate that a pre-drying step would be beneficial, though this is not modelled explicitly in the current version.

Material age (material_age)

The estimated duration of service before collection, expressed in years, serves as a proxy for cumulative thermal and UV degradation of the polymer chains. Prolonged outdoor exposure causes chain scission (reducing molecular weight), plasticizer migration out of the PVC matrix (reducing both flexibility and additive content), and surface chalking. Older materials exhibit lower tensile_strength_input values and produce recovered polymer with lower tensile strength in Routes 2 and 3, because the degradation of the base polymer chains cannot be reversed by the recycling process itself.

Input tensile strength (tensile_strength_input)

The ultimate tensile strength of the composite tarpaulin as received, before any shredding or processing. This variable integrates the contributions of PET fiber integrity, the degree of PVC chain degradation, and the residual plasticizer content. It is the primary predictor of the mechanical properties achievable in the recovered material for Routes 2 and 3: the tensile strength of the output polymer scales approximately proportionally with that of the input, modulated by output purity, as captured in the surrogate model equations.

# 3. Process Parameters

For each of the three recycling routes, six process parameters define the operating conditions that the NSGA-II optimizer is free to adjust within the bounds listed below. The parameters are route-specific and reflect the distinct physical or chemical mechanisms of each process.

## 3.1 Route 1 — Mechanical Recycling (T3.1)

Mechanical recycling separates PVC from PET through purely physical size reduction and classification: primary shredding, secondary milling, and electrostatic or density-based separation. No solvents or process heating above ambient temperature are required, making Route 1 the lowest-energy option for coarse polymer separation.

| Variable | Unit | Range | Description |
| --- | --- | --- | --- |
| shredder_speed | rpm | 200 – 1200 | Rotational speed of the primary shredder rotor blades. |
| mill_gap | mm | 0.1 – 5 | Clearance between the milling surfaces (rotor–stator gap) in the secondary size reduction step. |
| sieve_mesh_size | mm | 0.5 – 10 | Aperture size of the classification screen separating finished product from oversize material returned to the mill. |
| separation_cycles | n | 1 – 5 | Number of passes through the electrostatic or air-classification separation unit. |
| process_temp | °C | 20 – 80 | Ambient temperature of the processing environment or milling chamber. |
| throughput_rate | kg/h | 10 – 500 | Mass flow rate of material fed to the process line. |

Shredder speed: Higher speeds (>800 rpm) deliver finer primary fragments and improve subsequent PVC/PET liberation but increase electrical energy consumption—modelled as a super-linear function of speed (∝ speed1.5). Excessively high speeds risk fiber entanglement and frictional heat build-up, which can cause PVC softening and polymer co-mingling.

Mill gap: The primary determinant of output particle size. In the synthetic model: d50,out ≈ 0.55 × mill gap + 0.25 × sieve mesh size. Smaller gaps deliver finer powder with better PVC/PET liberation but increase mechanical wear and energy consumption per kilogram processed.

Sieve mesh size: Sets the upper bound on product particle size by controlling which particles pass through the classifier and which are returned to the mill. Larger apertures increase throughput and yield but allow co-mingled fragments to exit, reducing purity.

Separation cycles: Each additional pass through the electrostatic or density separator removes a further proportion of the minor polymer, improving purity at the cost of mass yield losses at each cycle.

Process temperature: Moderate warming (40–60 °C) slightly softens PVC and may improve mechanical liberation from PET fibers. Above 80 °C there is risk of thermal degradation and polymer sticking. This parameter has a minor secondary influence on purity in the current model.

Throughput rate: Higher throughput reduces unit electrical energy consumption per ton (denominator effect in kWh/t), but shortens residence time in the mill and separator, reducing both purity and yield.

## 3.2 Route 2 — Solvent / Texyloop Recycling (T3.2)

The Texyloop process selectively dissolves PVC in a hot organic solvent while leaving the PET scrim undissolved. PVC is then re-precipitated by cooling, washing, dried, and pelletized. The solvent is recovered in a closed loop. This route delivers the highest achievable PVC purity of the three routes (85–99.5%) at the cost of higher energy and solvent consumption.

| Variable | Unit | Range | Description |
| --- | --- | --- | --- |
| solvent_concentration | %vol | 10 – 80 | Volume fraction of the selective PVC solvent in the dissolution bath. |
| dissolution_temp | °C | 60 – 160 | Temperature of the dissolution vessel. |
| dissolution_time | min | 15 – 240 | Residence time of material in the dissolution vessel at the set temperature. |
| solid_liquid_ratio | g/L | 20 – 200 | Mass of shredded feedstock per litre of solvent in the dissolution vessel. |
| precipitation_temp | °C | 0 – 40 | Temperature to which the PVC-laden solution is cooled to re-precipitate PVC as a solid. |
| washing_cycles | n | 1 – 5 | Number of solvent-wash and water-rinse cycles applied to the precipitated PVC cake. |

Solvent concentration: Increasing the fraction of selective solvent (e.g., N-methyl-2-pyrrolidone, cyclohexanone, or proprietary Texyloop solvent) accelerates PVC dissolution kinetics and improves recovery but raises solvent consumption costs and the thermal energy required for downstream solvent recovery.

Dissolution temperature: The primary driver of PVC purity and PET recovery. Above ~100 °C, PVC dissolves rapidly in appropriate solvents while PET remains solid (differential solubility). Thermal energy consumption scales approximately linearly with this parameter and is the dominant energy cost of the route.

Dissolution time: Longer contact times allow complete PVC dissolution and cleaner PET separation, improving mass yield. Diminishing returns appear beyond ~120 minutes for well-pre-shredded feedstocks (d50 < 5 mm), where equilibrium dissolution is already approached.

Solid/liquid ratio: A higher ratio reduces solvent volume and cost per ton but increases solution viscosity, slows dissolution kinetics, and reduces PVC purity due to incomplete dissolution and higher carry-over of impurities into solution.

Precipitation temperature: PVC-laden solution is cooled to drive PVC re-precipitation. A larger temperature drop (high dissolution, low precipitation temperature) produces a finer, purer precipitate. Low precipitation temperatures require refrigeration, captured in thermal energy consumption.

Washing cycles: Each solvent wash and water rinse cycle removes residual solvent, degraded oligomers, and co-precipitated contaminants from the PVC cake, progressively improving purity. Net solvent consumed per ton depends on the joint effect of solvent concentration and the number of washing cycles.

## 3.3 Route 3 — Selective Additive Extraction (T3.3)

Route 3 targets the selective removal and recovery of plasticizers and stabilizers from the PVC matrix using a liquid extractant. Unlike Route 2, the PVC and PET polymers are not dissolved: they remain as solid substrates and emerge with a cleaner, higher-purity polymer matrix after the additives have migrated into the liquid phase.

| Variable | Unit | Range | Description |
| --- | --- | --- | --- |
| extractant_type | cat. | 0, 1, 2 | Chemical family of the extractant: 0 = polar organic solvent; 1 = enhanced polar with co-solvent; 2 = aqueous alkaline solution. |
| extractant_conc | %vol | 5 – 50 | Volume concentration of the extractant agent in the extraction bath. |
| extraction_temp | °C | 40 – 120 | Operating temperature of the extraction vessel. |
| extraction_time | min | 10 – 180 | Contact time of the material with the extractant. |
| ph_level | — | 2 – 12 | pH of the extraction bath, controlling selectivity between additive classes. |
| agitation_speed | rpm | 100 – 800 | Rotational speed of the extraction vessel agitator or tumbler. |

Extractant type: Three chemical families are modelled as a categorical variable. Type 0 — polar organic solvent (e.g., ethanol, acetone): good selectivity for phthalate and citrate plasticizers, baseline efficiency factor. Type 1 — enhanced polar solvent with co-solvent or surfactant: 15% higher efficiency factor in the model. Type 2 — aqueous alkaline solution: lower chemical efficiency but reduced environmental impact and cost. The type enters the model as a multiplicative factor on both plasticizer and stabilizer recovery.

Extractant concentration: Controls the chemical potential driving additive migration from the PVC matrix into the liquid phase. The relationship with recovery is non-linear (power-law exponent 0.7), reflecting equilibrium partitioning effects: doubling the concentration does not double the recovery.

Extraction temperature: Higher temperatures increase molecular diffusion coefficients (Arrhenius-type kinetics) and the solubility of plasticizers in the extractant, improving both the rate and equilibrium extent of extraction. Thermal energy consumption is the dominant energy cost of Route 3 and scales approximately linearly with temperature above the 40 °C baseline.

Extraction time: Controls the approach to extraction equilibrium. Short times (10–30 min) capture only surface-accessible additives. Times above ~120 min approach plateau recovery for typical particle sizes. Longer times increase both electrical consumption (agitation) and thermal consumption (temperature maintenance).

pH level: The pH of the extraction bath is the key selectivity parameter distinguishing plasticizer from stabilizer recovery. Plasticizer extraction is most efficient near neutral pH — the model represents this as a Gaussian efficiency factor centered at pH 7: e-0.1 × pH - 72. Stabilizer extraction (Ca/Zn metal soaps, organotin compounds) peaks in mildly alkaline conditions: e-0.08 × pH - 92. Selecting an intermediate pH introduces a fundamental trade-off between plasticizer and stabilizer recovery, making pH the primary selectivity control variable of this route.

Agitation speed: Controls the liquid-film mass transfer coefficient at the particle surface. Higher speeds thin the diffusion boundary layer and accelerate extraction kinetics. Electrical energy for agitation scales super-linearly with speed (∝ speed1.3).

# 4. Economic Parameters

All eight economic parameters are configurable at runtime from the Streamlit user interface or Jupyter notebooks. They are stored as fields of the EconomicParams dataclass and passed directly into the KPI equations — no additional model is needed to derive economic performance from physical outputs.

| Parameter | Default | Unit | Description |
| --- | --- | --- | --- |
| electricity_price | 0.12 | €/kWhₑₗ | Unit cost of electrical energy purchased from the grid. Applies to all three routes (shredder, agitator, pumps). Dominant cost for Route 1 (shredder/mill) and Route 3 (agitation). |
| thermal_energy_price | 0.06 | €/kWhₜₕ | Unit cost of thermal energy delivered to the process (steam or hot-oil circuit). Applies to Routes 2 and 3 only, where heating of dissolution or extraction vessels is required. |
| solvent_price | 1.50 | €/L | Net unit cost of make-up solvent consumed per ton of input, after in-process recovery and recycling. Applies to Route 2 only. |
| extractant_price | 2.00 | €/L | Unit cost of extractant chemical consumed per ton. Higher than solvent price per liter but consumed in smaller volumes. Applies to Route 3 only. |
| price_pvc_recycled | 400 | €/t | Market price of recycled PVC polymer meeting product quality specifications. Primary revenue driver for Routes 1 and 2. |
| price_pet_recycled | 350 | €/t | Market price of recycled PET fiber or pellet recovered as a co-product. Relevant for Routes 1 and 2. |
| price_plasticizer_recovered | 800 | €/t | Market value of recovered plasticizer (DINP, citrate esters). Significantly higher than bulk polymer value due to specialty chemical positioning. Route 3 only. |
| price_stabilizer_recovered | 1200 | €/t | Market value of recovered heat stabilizer (Ca/Zn complexes, organotin). Highest unit value among all co-products. Route 3 only. |

# 5. Data-Driven Surrogate Models

For each recycling route, one XGBoost regressor is trained per physical output variable. These surrogate models replace full process simulations: given any combination of material inputs and process parameters, a model prediction is returned in milliseconds, enabling the genetic algorithm to evaluate thousands of candidate operating points during an optimization run.

## 5.1 Feature Vector

For every route, the same 14-dimensional feature vector is assembled by concatenating the 8 material inputs (Section 2) and the 6 route-specific process parameters (Section 3):

x = x1, …, x88  material inputs,x9, …, x146  process parameters

Features are passed as raw numerical values with no standardization. XGBoost decision trees are invariant to monotone rescaling, so feature engineering is not required.

## 5.2 XGBoost Regressor Configuration

All 20 surrogate models share the same hyperparameter configuration:

| Hyperparameter | Value | Role |
| --- | --- | --- |
| n_estimators | 200 | Number of sequential boosting trees. Sufficient for the smooth, physically constrained correlations in the synthetic datasets. |
| max_depth | 5 | Maximum depth of each tree. Captures up to 5-way feature interactions; controls model complexity to avoid overfitting on 500-sample training sets. |
| learning_rate (η) | 0.05 | Shrinkage applied to each tree's contribution. A small value reduces variance at the cost of requiring more estimators. |
| subsample | 0.8 | Fraction of training rows sampled per tree (stochastic gradient boosting). Adds row-level randomness to reduce overfitting. |
| colsample_bytree | 0.8 | Fraction of features sampled per tree. Introduces feature-level diversity, analogous to Random Forest bagging. |
| random_state | 42 | Fixed seed for full reproducibility of training and predictions. |

## 5.3 Model Inventory

A total of 20 independent regressors are trained — one per physical output per route:

| Route | # Models | Predicted outputs |
| --- | --- | --- |
| R1 — Mechanical | 5 | pvc_purity, pet_purity, mass_yield, elec_consumption, particle_size_out_d50 |
| R2 — Solvent | 7 | pvc_purity, pet_recovery, mass_yield, elec_consumption, thermal_consumption, tensile_strength_output, solvent_consumed |
| R3 — Extraction | 8 | pvc_purity, mass_yield, elec_consumption, thermal_consumption, tensile_strength_output, plasticizer_recovery, stabilizer_recovery, additive_purity, extractant_consumed |
| Total | 20 | — |

Note that thermal_consumption is zero for Route 1 (no process heating) by construction in the synthetic data generator. The variables solvent_consumed and extractant_consumed are route-exclusive and zero when not applicable.

# 6. Technical and Economic KPIs

Five key performance indicators (KPIs) are computed analytically from the surrogate model outputs and the economic parameters. These KPIs form the five objective functions of the multi-objective optimization.

## 6.1 Operational Expenditure (OPEX)

OPEX represents the operating cost per ton of input material processed. It is the sum of energy and consumable costs:

OPEX eurot= Eel*pel+ Eth*pth+ Vsol*psol+ Vext*pext

Eel [kWh/t] and Eth [kWh/t] are the predicted electricity and thermal energy consumptions; Vsol [L/t] and Vext [L/t] are the solvent and extractant volumes consumed (zero when not applicable to the route); pel, pth, psol, psol are the corresponding unit prices from EconomicParams.

## 6.2 Revenue

Revenue is generated by the sale of recovered materials. Each co-product stream contributes proportionally to its recovery rate and the overall mass yield:

Revenue €t= mPVC × pPVC + mPET × pPET + mplast × pplast + mstab × pstab

where each recovered mass fraction mi  [dimensionless] is derived from the surrogate model predictions such as:

mi = recoveryi100× massyield100

Where recoveryi is the percentage recovery of co-product i predicted by the surrogate model (e.g., pvc_purity, pet_recovery, plasticizer_recovery, stabilizer_recovery). The term mass_yield is the overall percentage of input mass that exists as usable output. Co-products absent from a given route contribute zero revenue.

## 6.3 Net Margin

The net economic margin per ton of input is:

Net Margin €t= Revenue  -  OPEX

A positive net margin indicates that the route is economically profitable at the given prices and process conditions. This KPI is maximized as one of the five optimization objectives.

## 6.4 Total Energy Consumption

Total energy per ton combines electrical and thermal contributions:

Etot kWht= Eel + Eth

Eth = 0 for Route 1 by construction (no process heating). For Routes 2 and 3, Eth is predicted by a dedicated surrogate model and is the dominant energy cost. Total energy is minimized as one of the five optimization objectives.

## 6.5 Material Quality Proxy

Material quality represents the structural integrity of the recovered polymer. The proxy differs by route because Route 1 (mechanical recycling) does not dissolve or reform the polymer chains — no direct tensile strength measurement is applicable to its output.

Route 1 — fineness proxy. Finer output particles indicate better polymer liberation and a higher-quality product for downstream compounding. The quality metric is therefore the inverse of the output median particle size:

QR1 = 1d50,out + 0.01  mm-1

The constant 0.01 mm prevents division by zero at the minimum mill gap setting. Higher values of QR1 correspond to finer, better-liberated output — a more valuable product for compounding applications.

Routes 2 and 3 — tensile strength. For solvent and extraction routes, output polymer purity is high enough that tensile strength is a meaningful and directly predicted quality metric:

QR2, R3 = σtensile, out MPa

The tensile strength of the recovered polymer is predicted by a dedicated surrogate model, capturing the correlation with input tensile strength, output purity, material age, and (for Route 2) the number of washing cycles.

# 7. Multi-Objective Optimization — NSGA-II

The optimization problem is solved independently for each of the three recycling routes. For each route, the NSGA-II algorithm searches the 6-dimensional space of process parameters to simultaneously optimize all five KPIs, producing a set of non-dominated (Pareto-optimal) solutions.

## 7.1 Problem Formulation

For a given material input vector and economic price set, the optimization decision variable is the vector of 6 process parameters x ∈ R6, constrained within the per-route bounds defined in Section 3. The five-objective minimization problem is:

minfx=min   -PVC purity,   -Mass yield,   -Material quality,   Total energy,   -Net margin

Objectives that are to be maximized (purity, yield, quality, net margin) are negated so that the minimizer treats all five uniformly. Each function evaluation: (i) queries the 5–8 surrogate models with x; (ii) computes the KPIs from Section 6; (iii) assembles the five-component vector. No physical simulation is called — each evaluation is essentially instantaneous.

## 7.2 Algorithm Configuration

| Parameter | Value | Explanation |
| --- | --- | --- |
| Algorithm | NSGA-II | Non-dominated Sorting Genetic Algorithm II (Deb et al., 2002). The standard reference method for problems with 2–5 objectives. |
| Library | pymoo 0.6 | Open-source Python multi-objective optimization framework. |
| Objectives | 5 | PVC purity, mass yield, material quality, total energy, net margin (all simultaneously). |
| Decision variables | 6 per route | Route-specific process parameters (shredder speed, dissolution temperature, etc.). |
| Population size | 100 (adjustable) | Number of candidate solutions evaluated per generation. Range 20–200 in the Streamlit UI. |
| Generations | 200 (adjustable) | Number of evolutionary iterations. Range 20–500 in the UI. |
| Crossover | SBX (η=15, p=0.9) | Simulated Binary Crossover: generates offspring with the same distribution as a single-point crossover applied to real-valued variables. |
| Mutation | PM (η=20) | Polynomial Mutation: a bounded perturbation operator for real-valued decision variables. |
| Termination | Fixed generations | Algorithm stops after the prescribed number of generations. |
| Seed | 42 | Fixed random seed ensuring reproducible Pareto fronts across runs. |

# 8. Best Route Selection — Hypervolume Indicator

After the three independent NSGA-II runs are complete, the results are compared to recommending the single best recycling route. This comparison uses the hypervolume indicator (also called the S-metric or dominated hypervolume), a rigorous, single-scalar summary of the quality and diversity of each route’s Pareto-optimal front.

## 8.1 Context: The Pareto Front as a Geometric Object

The output of NSGA-II for each route is not a single optimal solution but a Pareto-optimal front— a set of N solution vectors in the 5-dimensional objective space:

Fk =  f1, f2, …, fN  ⊂  R5,     k ∈  R1, R2, R3

A solution fa dominates fb if it is at least as good in all five objectives and strictly better in at least one. No solution on the Pareto front is dominated by any other within that front — every point on the front represents a legitimate trade-off: improving one objective (e.g. PVC purity) necessarily worsens at least one other (e.g. energy or margin).

In practical terms, the shape of the front reveals the structural freedom the route offers: a wide, spread-out front means the process has many levers the engineer can use to trade independently between objectives; a narrow or clustered front means the objectives are strongly coupled and little room exists to change one without affecting the others.

## 8.2 Normalization

The three fronts are expressed in different physical units and scales. To allow a meaningful geometric comparison, all points from all three fronts are pooled and each objective dimension is rescaled to the interval [0, 1]:

Fi,j=  Fi,j  -  fminj  fmaxj  -  fminj +  ε

fminj and fmaxj are the global minimum and maximum of objective j across all three routes; ε = 10-9 prevents division by zero. After normalization, every objective component lies in [0, 1] and the comparison is dimensionless: no route is favored because its objectives happen to span a larger numerical range.

## 8.3 The Reference Point

A reference point r = 1.1, 1.1, 1.1, 1.1, 1.1 is placed at 1.1 in each normalized dimension, which is strictly worse than any observed solution in all five objectives simultaneously. It defines the “bad corner” of the evaluation region. The 0.1 buffer beyond the maximum normalized value (1.0) ensures that every solution, even the weakest, contributes a positive volume to the hypervolume calculation.

## 8.4 Geometric Meaning of the Hypervolume

### Plain-language explanation

The hypervolume measures how much of the ‘good zone’ a route’s Pareto solutions collectively cover in objective space.

Step 1 — Think in two dimensions first. Imagine plotting each Pareto solution as a dot on a chart with PVC purity on one axis and net margin on the other (after negation, both axes point in the ‘good’ direction). The reference point r sits at the ‘bad corner’ — lowest purity, lowest margin.

For each dot, draw a rectangle that extends from that dot all the way to the bad corner. This rectangle represents every combination of purity and margin that is at least as good as that dot — its zone of dominance. Now take the union of all rectangles from all the dots on the front: the total covered area is the hypervolume in two dimensions.

A route whose solutions push far from the bad corner (high purity AND high margin simultaneously) produces large rectangles and covers a large area. A route whose solutions cluster near the bad corner covers very little. Larger area = better route. The hypervolume measures how much of the ‘good territory’ the route’s solutions collectively dominate.

Step 2 — Extend to five dimensions. The RETAIN DSS has five objectives. The chart now has five axes instead of two. Each solution defines a 5-dimensional box stretching from the solution to the bad corner. The hypervolume is the total 5-dimensional volume of the union of all those boxes.

Step 3 — Interpretation. A route with a larger hypervolume offers a better and more balanced set of trade-offs across all five performance criteria simultaneously. A route that excels in one objective but is weak in the other four produces a thin, flat slab of volume. A route that is good in all five at once produces large, fat boxes — and dominates a much greater region of objective space.

### Technical formulation

Each normalized Pareto solution fi∈Fk defines an axis-aligned hyperrectangle with corners at fi and r. The hypervolume is the Lebesgue measure (volume) of the union of all such hyperrectangles:

HV Fk, r  =  λ5f∈ Fk f, r

where λ5 denotes the 5-dimensional Lebesgue measure and [f, r ] is the axis-aligned hyperrectangle with corners f and r. Overlapping boxes are counted only once (union, not sum), so the hypervolume is not inflated by clustered solutions. Each additional non-dominated solution can only maintain or increase the hypervolume rate, it is a monotone quality indicator.

The hypervolume has three desirable properties as a multi-objective quality indicator: (i) Pareto-compliance — it is strictly larger for a Pareto-dominating front; (ii) scale-invariance — addressed by the normalization step; (iii) sensitivity to both convergence and spread — it rewards simultaneously getting close to the ideal point and having a well-distributed, diverse front.

## 8.5 Route Recommendation and Representative Solution

The route with the largest hypervolume is selected as the recommended route:

k*=  argmaxk HV( Fk, r )

A larger hypervolume means the route’s Pareto front dominates a larger portion of the normalized objective space — that is, it achieves better simultaneous performance across more of the five KPI dimensions than the competing routes, given the specific feedstock properties and market prices provided by the user.

Once the recommended route k* is identified, a single representative operating point is reported to the process engineer. Among all Pareto solutions of route k*, the solution with the highest PVC purity is selected (minimum in the negated F[:, 0] column), because PVC purity is the primary product quality specification in all tarpaulin recycling contexts. The associated 6-dimensional process parameter vector x* is displayed in the ‘Optimal parameters’ table of the Results page.

This representative point is not globally best across all five objectives simultaneously — no such point exists on a true Pareto front. It is the best achievable trade-off along the PVC purity axis, chosen as the most commercially relevant anchor for the process engineer’s operating decision.

—  End of Technical Reference  —